REPUBLIC OF ARMENIA

MINISTRY OF LABOR AND SOCIAL AFFAIRS

SOCIAL PROTECTION ADMINISTRATION II PROJECT

LOAN N 5398-AM

ENVIRONMENTAL AND SOCIAL MANAGEMENT PLAN

RENOVATION OF PREMISES

FOR KAPAN COMPLEX SOCIAL SERVICES TERRITORIAL CENTER

AUTHORS:

Engineer/Environmental specialist – T. Tadevosyan

ME and Social Safeguards specialist – N. Mkrtchyan

JULY 2019

PART A: General Project and Site Information

| INSTITUTIONAL & ADMINISTRATIVE |  |  |  |  |
| --- | --- | --- | --- | --- |
| Country | Republic of Armenia |  |  |  |
| Project title | Social Protection Administration II Project (SPAP II), Renovation of Premises for Kapan Complex Social Services Territorial Center (CSSTC) |  |  |  |
| Scope of site-specific activity | The following activities will be implemented at the construction site of Kapan CSSTC: restoration of the roof and water drainage system, replacement of internal engineering networks (utilities) and arranging external connections, installing internal and external doors and windows, demolition of the existing non-bearing walls and construction of new walls, finishing of internal walls and ceilings, mounting of internal infrastructure, construction of wheelchair ramp, partial yard improvement works (renovation of staircases, tiling, front door canopy, replacement or restoration of drainage (wastewater disposal) pipes. The first floor of the building with an area of 522.86 sq. m and an area of 235 sq. m from the second floor have been allocated for the establishment of the CSSTC (7,33 sq. m staircase area). |  |  |  |
| Institutional arrangements (WB) | Task Team Leaders: Ivan Drabek, Marina Petrovic |  | Safeguards Specialists: Darejan Kapanadze, Environment Vera Dugandzic, Social |  |
| Implementation arrangements (RoA) | Implementing entity: Ministry of Labor and Social Affairs (MLSA) | Works supervisor: MLSA: Tovmas Tadevosyan, Environmental Nvard Mkrtchyan, Social |  | Works contractor: (TBD) |
| SITE DESCRIPTION |  |  |  |  |
| Name of institution whose premises are to be rehabilitated | Kapan Complex Social Services Territorial Center (CSSTC) in Syunik Marz |
| Address and site location of institution whose premises are to be rehabilitated | M. Stepanyan str. 13/5, Kapan, Syunik Marz, Republic of Armenia |  |  |  |
| Who uses the land (formal/informal)? | The land is owned by Kapan community. The document confirming the real estate acquisition right is the Certificate of State Registration of the Rights over Real Estate N 09-0024, issued 31.10.2018 provided on the basis of the RoA Government Decree N131-A, dated 15.02.2018, and property delivery-acceptance Act, dated 28.09.2018 (Appendix 2). The premises allocated for the establishment of the CSSTC are in Kapan Community medical center administrative building. The premises were used by the medical center personal. However currently the premises are not used in any way as the medical center moved into a new building. Thus, the renovation works do not entail any physical or economic displacement. |  |  |  |
| Description of physical and natural environment around the site | The building is in the central part of Kapan community. There are state institutions, residential, other buildings and bank branches nearby. The road leading to the Center is asphalted, there are no trees and bushes hindering the construction works. The external electricity, natural gas, water supply and wastewater disposal of the CSSTC are currently in place and will remain unchanged; and the internal communications and infrastructures, electricity wires and water pipes, will be fully replaced. The premises are State property and cannot be used for private reasons or profit. The civil works will not interfere with everyday life of nearby residents or the activity of the public transportation. The renovation works will take 210 days from the start of works. Project’s Social and Environmental Specialists have visited and pre-screened the site. |  |  |  |
| Locations and distance for material sourcing, especially aggregates, water, stones? | The necessary aggregates and other construction materials are available from nearby vendors or wholesale trade centers. Water required for construction works is available inside the building, supplied by “Veolia Jur” CJSC. There is sufficient space for parking construction vehicles and machinery on the territory adjacent to CSSTC and to establish a temporary warehouse for the building materials and construction waste if necessary. |  |  |  |
| LEGISLATION |  |  |  |  |
| National & local legislation & permits that apply to project activity | The following Armenian legislation defines a legal framework applicable to project activities: - Law on Atmospheric Air Protection of RoA (1994). The purpose of Law on Atmospheric Air Protection is to define main principles of the RoA, directed to provision of purity of atmospheric air and improvement of air quality, prevention and mitigation of the chemical, physical, biological and other impacts on air quality and regulation of public relation, as well as the principle of setting standards of emission limits on atmospheric air pollutants and on permissible physical harmful influences. According to this law contractor shall undertake repairs activities as well as transportation and temporary storage of wastes the way to minimize dust and other emissions to the air. - RoA Law on Waste (2004). The law provides the legal and economic basis for collection, transportation, disposal, treatment, re-use of wastes as well as prevention of negative impacts of waste on natural resources, human life and health. The law defines the roles and responsibilities of the state authorized bodies as well as of waste generator organizations in waste management operations. According to this law, the waste generated during renovation of the building should be recycled as appropriate or disposed of in designated locations. According this law, the contractor shall absorb costs of safe storage, transportation and disposal of the waste. - RoA Law on Environmental Impact Assessment and Expertise (2014) The law defines types of activities which are subject to environmental impact assessment and ecological expertize. According to this law, works for the rehabilitation of premises for the CSSTC are not subject to the ecological expertize. - Land Code of the Republic of Armenia (2001) Taking into account the nature protection, economic and social significance of the land, the Land Code defines the basic directions of State regulatory system concerning land relations, development of various organizational and legal forms of land economy, fertility of land, land use efficiency raise, protection and improvement of an environment – favorable for human vitality and health, the legal framework concerning the protection of the rights on land, as well as regulation of resettlement and evacuation problems. Rehabilitation of Kapan CSSTC will not cause resettlement neither as defined by this law nor as by OP/BP 4.12 of the World Bank. - Water Code of the Republic of Armenia (2002) Water Use: Removing water from or otherwise reducing water in a water resource; storing water; impeding or diverting the flow of a water resource; polluting a water resource; discharging wastewater into a water resource; disposing or storing hazardous substance in a manner that may detrimentally impact a water resource; and, altering the beds, banks, course or characteristics of a water resource. Water Use Permit: A document, which includes the rights and responsibilities of a person related to the extraction of water from and discharge of water into a water resource. The Design Company shall secure a water use permit for any required type of water use, except cases determined by this Code. - RoA Law on Nature Protection and Nature Utilization (1998) Nature protection payment – compulsory payment that generates funds for implementation of environmental programs which, under this law, is paid to the State budget or, in cases specified by law, to the community budgets. The Construction Company that implements activities specified in this law is responsible for nature protection and nature utilization payments. - RoA Law on Urban Development (1998). The law defines types of activities which are subject to construction permitting. According to this law, rehabilitation works (except for construction, reconstruction) planned at institution do not require obtaining of the construction permit. |  |  |  |
| PUBLIC CONSULTATION |  |  |  |  |
| When / where the public consultation process will take /took place | Present draft ESMP will be disclosed through the webpage of MLSA (www.mlsa.am) in Armenian and English languages upon clearance by the World Bank. Public consultation on the draft ESMP will be announced through the media accessible for local stakeholders and held after disclosure of the draft ESMP. Brief information on the planned works and contact information of contractor for addressing questions and grievance will be placed at the work site and/or in its immediate surroundings. |  |  |  |
| ATTACHMENTS |  |  |  |  |
| Attachment 1: Site plan/photos of the construction site Attachment 2: Ownership certificate of the premises Attachment 3: Conclusion of the seismic resistance test Attachment 4: Minutes of public consultation meeting and announcement-invitation (to be provided) Attachment 5: Agreement for the waste disposal |  |  |  |  |

PART B: safeguards information

| ENVIRONMENTAL /SOCIAL SCREENING |  |  |  |
| --- | --- | --- | --- |
| Will the site activity include/involve any of the following? | Activity/Issue | Status | Triggered Actions |
|  | - Building rehabilitation | [] Yes [ ] No | See Section A below |
|  | - New construction | [ ] Yes [] No | See Section A below |
|  | - Individual wastewater treatment system | [ ] Yes [] No | See Section B below |
|  | - Historic building(s) and districts | [ ] Yes [ ] No | See Section C below |
|  | - Acquisition of land | [ ] Yes [] No | See Section D below |
|  | - Hazardous or toxic materials | [ ] Yes [] No | See Section E below |
|  | - Impacts on forests and/or protected areas | [ ] Yes [] No | See Section F below |
|  | - Handling / management of medical waste | [ ] Yes [] No | See Section G below |
|  | - Traffic and Pedestrian Safety. | [ ] Yes [] No | See Section H below |

PART C: Mitigation measures

| ACTIVITY | PARAMETER | MITIGATION MEASURES CHECKLIST |
| --- | --- | --- |
| General Conditions | Notification and Worker Safety | - The local construction and environment inspectorates and communities have been notified of upcoming activities - The public has been notified of the works through appropriate notification in the media and/or at publicly accessible sites (including the site of the works) - All legally required permits have been acquired for construction and/or rehabilitation - The Contractor formally agrees that all work will be carried out in a safe and disciplined manner designed to minimize impacts on neighboring residents and environment. - Workers’ PPE will comply with international good practice (always hardhats, as needed masks and safety glasses, harnesses and safety boots) - Appropriate signposting of the sites will inform workers of key rules and regulations to follow. |
| A. General Rehabilitation and /or Construction Activities | Air Quality | - Demolition debris shall be kept in controlled area and sprayed with water mist to reduce debris dust - During pneumatic drilling/wall destruction, dust shall be suppressed by water spraying and/or installing dust screen enclosures at site - The surrounding environment (sidewalks, roads) shall be kept free of debris to minimize dust - There will be no open burning of construction / waste material at the site - There will be no excessive idling of construction vehicles at sites |
|  | Noise | - Construction noise will be limited to restricted times agreed to in the permit. The civil works will cause some noise mostly in defined working hours, however the Contractor shall maximally reduce inconvenience caused to the nearby residents - During operations the engine covers of generators, air compressors and other powered mechanical equipment shall be closed, and equipment placed as far away from residential areas as possible |
|  | Water Quality | - The site will establish appropriate erosion and sediment control measures such as e.g. hay bales and / or silt fences to prevent sediment from moving off site and causing excessive turbidity in nearby streams and rivers. |
|  | Waste management | - Waste collection and disposal pathways and sites will be identified for all major waste types expected from demolition and construction activities. - Mineral construction and demolition wastes will be separated from general refuse, organic, liquid and chemical wastes by on-site sorting and stored in appropriate containers. - Construction waste will be collected and disposed properly by licensed collectors - The records of waste disposal will be maintained as proof for proper management as designed. - Whenever feasible the contractor will reuse and recycle appropriate and viable materials (except asbestos) |
| H. Traffic and Pedestrian Safety | Direct or indirect hazards to public traffic and pedestrians by construction activities | In compliance with national regulations the contractor will insure that the construction site is properly secured and construction related traffic regulated. This includes but is not limited to - Signposting, warning signs, barriers and traffic diversions: site will be clearly visible and the public warned of all potential hazards - Traffic management system and staff training, especially for site access and near-site heavy traffic. Provision of safe passages and crossings for pedestrians where construction traffic interferes. - Adjustment of working hours to local traffic patterns, e.g. avoiding major transport activities during rush hours or times of livestock movement - Active traffic management by trained and visible staff at the site, if required for safe and convenient passage for the public. - Ensuring safe and continuous access to office facilities, shops and residences during renovation activities, if buildings stay open for public. |

PART D: Monitoring Plan

| Activity | What (Is the parameter to be monitored?) | Where (Is the parameter to be monitored?) | How (Is the parameter to be monitored?) | When (Define the frequency / or continuous?) | Why (Is the parameter being monitored?) | Who (Is responsible for monitoring?) |
| --- | --- | --- | --- | --- | --- | --- |
| CONSTRUCTION PHASE |  |  |  |  |  |  |
| Provision of construction materials | Purchase of construction materials from the registered providers | Provider’s office or warehouse | Verification of labels of the materials and/or certificates | During conclusion of supply contracts | Ensure reliability of construction materials and their safety for human health | MLSA |
| Generation of construction waste | - Temporary storage of construction waste in especially allocated areas; - Timely disposal of wastes to the formally designated locations | Work site | Inspection | Periodically during reconstruction and upon its completion | - Prevent pollution of territory - avoid accidents at construction site due to scattered fragments of construction materials and debris - retain esthetic appearance of the reconstruction site and other floors | MLSA |
| Transportation of construction materials and waste | - Technical condition of vehicles and machinery; - Confinement and protection of truck loads with lining; - Respect of the established hours and routes of transportation | - Construction site - Routes of transportation of construction materials and wastes | Inspection of roads adjacent to the construction object in the direction of the movement route | Selective inspections during work hours | - Limit pollution of soil and air from emissions; - Limit nuisance to local communities from noise and vibration; - Minimize traffic disruption. | Traffic Police |
| Workers’ health and safety | - Provision of constructors with working clothes and PPE; - Strict compliance with the rules of construction equipment operation and usage of PPE; - Presence of fire extinguishing and medical aid kits at work site | Work site | Inspection | Entire period of works | Reduce probability of traumas and accidents | MLSA |
| Traffic and road safety | - Presence of traffic regulation and warning signs as needed; - Construction site demarcated/fenced, and all construction activities confined to the site; - No construction materials and waste piled up or scattered outside the construction site | Adjacent area to the construction site | Inspection | Entire period of construction works | Prevent the violation of interests of the passengers and/or pedestrians because of the construction works | MLSA |
| Site reinstatement | Land plot around the reconstructed premises cleared of any residual waste and reinstated | Construction site | inspection | Final stage of works | Ensure complete close-down of the construction site with no residual trash left behind | MLSA |
| Public information and grievance redress | Information board installed at the work site carrying: - the name of works provider (contractor), works commissioner (MLSA), and works financing Project (SPAP); - contact information / hotline in MLSA for local communities to refer complains on the nuisance caused by construction works | Field office of the contractor and construction site | Inspection of documents | Before and during the construction works | Communicate with the residents or employees as well as other stakeholders of the area adjacent to the construction site and register the complaints | MLSA |
| OPERATION PHASE |  |  |  |  |  |  |
| Operational waste management | - Organized collection of household waste generated during office operation inside CSSTC; - Arrangements in place and operational for removal of household waste from CSSTC | CSSTC space | Inspection of office area | Entire period of office operation | Ensure sanitary safety and favorable conditions for CSSTC staff | CSSTC administration |
| Maintenance of internal communication systems | Electric wiring and water/waste water pipes maintained in good operation condition | CSSTC space | Inspection of office area | Entire period of office operation | Exclude risks of electric shocks, short circuits, and leakage of pipes | CSSTC administration |
| Improved space (premises) | Exterior area of the building kept clean and groomed | Premises of the CSSTC | Inspection | Entire period of works implementation | Ensure favorable social impact of the improved premises on the beneficiaries attending the center. | CSSTC administration |

Attachment 1. Site plan and

photo documentation of premises before repair

Attachment 2

Ownership Certificate of the Building

(Non-official translation)

Republic of Armenia

Certificate of State Registration of the Rights over

Real Estate

This Certificate confirms the state registration of the rights over real estate in the integrated registry of state registration of rights over real estate on October 31, 2018 by the following details:

1. Entity(s) of registered right

Ministry of Labor and Social Affairs of the Republic of Armenia

2. The place and name of the real estate

M. Stepanyan str. 13/5, Kapan Community, Syunik Marz

3. Documents on which the registration is based:

RoA Government Decree N131-A, dated 15.02.2018, and Property Delivery-Acceptance Act, dated 28.09.2018

4. Land characteristics

Cadaster Code 09-001-0617-0006

Surface ha 1.8695

Target use Residential

Operational use Public Construction

Type of registered right Common shared property

5. Building characteristics

1) Target use – public

2) Characteristics of the separate buildings;

| N | Cadaster Code | Type | Surface | Type of registered right |
| --- | --- | --- | --- | --- |
| 1 | 09-001--0617-0006-009 | Building | 765.19 sq. m | Ownership |
| 2 |  |  |  |  |
| 3 |  |  |  |  |

Additional Notes and Information

The allocated property is attached to the RoA MLSA state institution.

Registering official – Ruzanna Asryan

Position – Official at Real estate register of Syunik Marz subdivision of the State Committee of the Real Estate Cadaster adjunct to the RoA Government.

Certificate number-31102018-09-0024, password – QXF4RLDKQNRJ

Attachment 3.

Conclusion of the Seismic Resistance Test

SUMMARY-EXCERPT FROM THE CONCLUSION

on the Evaluation of the Technical State and Seismic Vulnerability of Premises

Allocated for KAPAN Community Complex Social Services Territorial Center in Syunik Marz

This Conclusion is provided by “Husali Kamar” LLC licensed company (license N 13763) on the basis of the Contract N19/02 concluded with the relevant Design Company on 15.01.2019.

Based on the visual examination the technical state of the specified building is evaluated as satisfactory.

Taking into account the characteristics of actual architectural volume-plan and constructive solutions of

Kapan community CSSTC, as well as the actual technical state and the degree of damage it is recommended to implement the following activities to raise the seismic resistance of the building.

- Overall re-planning of the building as the actual constructive solutions of the partitions do not meet the necessary construction norms and requirements and are not compliant with internal planning solutions.

- Restoration of the roof and water drainage system,

- Restoration/ replacement of internal engineering networks (utilities/water supply, sewage system, heating system, electricity, ventilation) and arrangement of external connections,

- Restoration of the stones on the external walls of the building where some of the stones have been displaced and fallen.

- Restoration, renovation of the external frontal walls and covering with external waterproof paint.

- Replacement of windows.

- Replacement of internal doors.

- Renovation of floors and ceilings in compliance with acting construction norms.

- Yard improvement works, including renovation of staircases, tiling, construction of wheelchair ramp, front door canopy.

- Replacement or restoration of drainage (wastewater disposal) pipes.

“Husali Kamar” LLC Director                  (sign/seal)                            V. Arakelyan

Attachment 4. Records of Public Consultation

Attachment 5.

Agreement for Waste Disposal

KAPAN COMMUNITY ADMINISTRATION OF THE

ROA SYUNIK MARZ

Waste Management Permit N27 issued 19.06.2019

SUMMARY-EXCERPT from the ISSUED PERMIT

The construction waste generated during the renovation works at the construction site of Kapan Complex Social Services Territorial Center will be removed and disposed off at a dump in Syunik village about 12,2 km away from the construction site maintaining all the safety measures, environmental norms in compliance with the RoA legislation.

Deputy Head of Kapan Community Administration    sign/seal    Davit Hayrapetyan

Land acquisitions includes displacement of people, change of livelihood encroachment on private property this is to land that is purchased/transferred and affects people who are living and/or squatters and/or operate a business (kiosks) on land that is being acquired.

Toxic / hazardous material includes but is not limited to asbestos, toxic paints, noxious solvents, removal of lead paint, etc.